ENGR 12				Assignment 10			 Due: next wed

Part I.  Drills  -- 1 point each

For questions 1 and 2, assume  a = 3 – 4j,  b =  –6j,  c = 8<-45 o, d = 15<250o

| - Express the following in Rectangular form: - (a + b) - ( a / b) - c - c* (complex conjugate of c) - (c + d) | - Express the following in Polar form: - b - b* (that's the complex conjugate of b) - c/d - a b - a + d |
| --- | --- |

- Complete the missing cells of the table defining three sinusoids and express each as 
             it= Im cos⁡(ωt+ φ)

- Use the concept of a phasor to convert the following expressions into a simpler form:

  - 100cos(500t – 43 o) + 45cos(500t + 120 o)

  - 200cos(377t + 75 o) + 50sin(377t – 100 o)

  - 5cos(wt) + 5cos(wt+120 o) + 5cos(wt-120 o)

- Find the complex impedance AND admittance of the following at a-b, given w = 5000 Radians/sec, express in both polar and rectangular form:
a)									b)

Part II.  Assisted Problem Solving – 2 pts

| - Find the the current through the 25nF cap in 5a) when a current source of 10cos(5000t + 120o) is applied at a-b. Express your answer in both phasor and time-domain form | Plan - Convert the source current into phasor form - Use the current divider formula to find phasor Ic - Convert phasor Ic to time domain |
| --- | --- |
| - Find the voltage across the 2.5mF cap in 5b) when a voltage source of 50cos(5000t – 90 o) is applied at a-b. Express your answer in both phasor and time-domain form | PLAN - Convert the source voltage into phasor form - Use the voltage divider formula to find phasor Vc - Convert phasor Vc to time domain |

Part III. Unassisted Problem Solving – 3 points

8)  What value of w will the impedance at a-b in problem 5b become purely resistive?